- CDG Stage 1 Test

1. System Configuration

Figure 1. System Configuration

The C2K-ATS (CDMA 2000-Automatic Test System) provides an integrated test solution for evaluating parametric performance and signaling conformance of CDMA mobile devices. By combining SR 5500 RF Channel Emulator, TAS 5600 Noise & Interference Emulator, and Air Access C2K (TAS3450 and TAS5200) with a base station emulator (E5515C) and the other essential CDMA test equipments, a turn-key test system is used. TASKIT/C2K Windows software automates mobile testing by configuring the instruments, stepping through the test sequences, and logging the results.

Table 1. Component and Function

| Component | Function |
| --- | --- |
| ① Agilent E4404B | System calibration, System diagnostics, and Transmitter measurements of the mobile station. |
| ② Agilent 8960 | Base Station Emulator |
| ③ TAS 3450 | Providing multiple sectors for true soft, softer and hard handoff emulation. |
| ④ TAS 5200 |  |
| ⑤ GSS 5060 | AFLT/A-GPS/GPS/Hybrid Position Location Test |
| ⑥ TAS 5048 | Switching of RF connections |
| ⑦ TAS 5068 | Switching of RF connections |
| ⑧ SR 5500 | Multi Path Fading Emulator |
| ⑨ TAS 5600 | Dual tone CW generator |
| ⑩ Agilent 6630B | DC Power Source |
| ⑪ TAS 3710 | AFLT/A-GPS/GPS/Hybrid Position Location Test |

2. Service

- Development Confirmation testing

- CTIA/CCF Certification testing

- TRAC A Certification testing

- Performance Quality testing for domestic mobile phones

3. Test Coverage

Table 2. Test Coverage (C.S0011)

| C.S0011 | Recommended Minimum Performance Standards for cdma2000 Spread Spectrum Mobile Stations | TTA Coverage |
| --- | --- | --- |
| 3 | CDMA RECEIVER MINIMUM STANDARDS |  |
| 3.1 | Frequency Coverage Requirements |  |
| 3.2 | Acquisition Requirements |  |
| 3.2.1 | Idle Handoff Tests |  |
| 3.2.1.1 | Idle Handoff in non-Slotted Mode on the Paging Channel | feasible |
| 3.2.1.2 | Idle Handoff in Slotted Mode on the Paging Channel | feasible |
| 3.2.1.3 | Idle Handoff in Slotted Mode on the Forward Common Control Channel |  |
| 3.2.1.4 | Idle Handoff to Another Frequency |  |
| 3.2.2 | Soft Handoff Tests |  |
| 3.2.2.1 | Neighbor Set Pilot Detection and Incorrect Detection in Soft Handoff | feasible |
| 3.2.2.2 | Candidate Set Pilot Detection and Incorrect Detection in Soft Handoff | feasible |
| 3.2.2.3 | Active Set Pilot Loss Detection in Soft Handoff | feasible |
| 3.2.3 | Access and Access Probe Handoff Tests |  |
| 3.2.3.1 | Access Probe Handoff |  |
| 3.2.3.2 | Access Handoff |  |
| 3.2.4 | Candidate Frequency Single Search | feasible |
| 3.3 | Forward Common Channel Demodulation Performance |  |
| 3.3.1 | Demodulation of non-Slotted Mode Paging Channel | feasible |
| 3.3.2 | Demodulation of Slotted Mode Paging Channel | feasible |
| 3.3.3 | Demodulation of Broadcast Control Channel in AWGN Channel |  |
| 3.3.4 | Demodulation of Broadcast Control Channel in Multipath Fading Channel |  |
| 3.3.5 | Demodulation of Forward Common Control Channel |  |
| 3.3.6 | Demodulation of Common Assignment Channel and Reception of Common Power Control Channel |  |
| 3.4 | Forward Traffic Channel Demodulation Performance |  |
| 3.4.1 | Demodulation of Forward Traffic Channel in Additive White Gaussian Noise | feasible |
| 3.4.2 | Demodulation of Forward Traffic Channel in Multipath Fading Channel | feasible |
| 3.4.3 | Demodulation of Forward Traffic Channel During Soft Handoff | feasible |
| 3.4.4 | Decision of Power Control Bit for Channels Belonging to Different Power Control Sets During Soft Handoff | feasible |
| 3.4.5 | Decision of Power Control Bit for Channels Belonging to the Same Power Control Set | feasible |
| 3.4.6 | Demodulation of Power Control Subchannel During Soft Handoff |  |
| 3.4.7 | Demodulation of Forward Traffic Channel in Multipath Fading Channel with Closed Loop Power Control (FPC_MODE='000') | feasible |
| 3.4.8 | Demodulation of Forward Traffic Channel in Multipath Fading Channel with Closed Loop Power Control (FPC_MODE='010') |  |
| 3.4.9 | Demodulation of Forward Traffic Channel in Multipath Fading Channel with Outer Loop Power Control and Closed Loop Power Control (FPC_MODE='000', '001' and '010') | feasible |
| 3.4.10 | Demodulation of Forward Traffic Channel in Multipath Fading Channel with Closed Loop Power Control (FPC_MODE='000') and Transmit Diversity (OTD or STS) |  |
| 3.4.11 | Demodulation of Forward Traffic Channel in Multipath Fading Channel with Closed Loop Power Control (FPC_MODE='010') and Transmit Diversity (OTD or STS) |  |
| 3.4.12 | Demodulation of Power Control Subchannel During Reverse Pilot Channel Gating |  |
| 3.4.13 | Demodulation of Power Control Subchannel During Reverse Fundamental Channel Gating |  |
| 3.5 | Receiver Performance |  |
| 3.5.1 | Receiver Sensitivity and Dynamic Range | feasible |
| 3.5.2 | Single Tone Desensitization | feasible |
| 3.5.3 | Intermodulation Spurious Response Attenuation | feasible |
| 3.5.4 | Adjacent Channel Selectivity |  |
| 3.5.5 | Receiver Blocking Characteristics |  |
| 3.6 | Limitations on Emissions |  |
| 3.6.1 | (Rx) Conducted Spurious Emissions | feasible |
| 3.6.2 | (Rx) Radiated Spurious Emissions |  |
| 3.7 | Supervision |  |
| 3.7.1 | Paging Channel | feasible |
| 3.7.2 | Forward Traffic Channel |  |
| 4 | CDMA TRANSMITTER MINIMUM STANDARDS |  |
| 4.1 | Frequency Accuracy | feasible |
| 4.2 | Handoff |  |
| 4.2.1 | CDMA to CDMA Hard Handoff | feasible |
| 4.2.2 | Transmit Power After Hard Handoff | feasible |
| 4.3 | Modulation Requirements |  |
| 4.3.1 | Time Reference | feasible |
| 4.3.2 | Reverse Pilot Channel to Code Channel Time Tolerance | feasible |
| 4.3.3 | Reverse Pilot Channel to Code Channel Phase Tolerance | feasible |
| 4.3.4 | Waveform Quality and Frequency Accuracy | feasible |
| 4.3.5 | Code Domain Power | feasible |
| 4.4 | RF Output Power Requirements |  |
| 4.4.1 | Range of Open Loop Output Power | feasible |
| 4.4.2 | Time Response of Open Loop Power Control | feasible |
| 4.4.3 | Access Probe Output Power | feasible |
| 4.4.4 | Range of Closed Loop Power Control | feasible |
| 4.4.5 | Maximum RF Output Power | feasible |
| 4.4.6 | Minimum Controlled Output Power | feasible |
| 4.4.7 | Standby Output Power and Gated Output Power | feasible |
| 4.4.8 | Power Up Function Output Power |  |
| 4.4.9.2.1 | Code Channel Output Power for the Enhanced Access Channel Header, Enhanced Access Channel Data, and Reverse Common Control Channel Data |  |
| 4.4.9.2.2 | Code Channel Output Power for the Reverse Traffic Channel | feasible |
| 4.4.10 | Reverse Pilot Channel Transmit Phase Discontinuity |  |
| 4.4.11 | Reverse Traffic Channel Output Power During Changes in Data Rate |  |
| 4.5 | Limitations on Emissions |  |
| 4.5.1 | (TX) Conducted Spurious Emissions | feasible |
| 4.5.2 | (TX) Radiated Spurious Emissions |  |
| 4.5.3 | Occupied Bandwidth |  |
| 5 | Environmental Requirements |  |
| 5.1 | Temperature and Power Supply Voltage | feasible |
| 5.2 | High Humidity | feasible |
| 5.3 | Vibration Stability | feasible |
| 5.4 | Shock Stability | feasible |

Table. Test Coverage (C.S0033)

| C.S0033 | Recommended Minimum Performance Standards for cdma2000 High Rate Packet Data Access Terminal | TTA Coverage |
| --- | --- | --- |
| 3.1.1.1 | Frequency Coverage Requirements |  |
| 3.1.1.2.1 | Demodulation of Forward Traffic Channel in AWGN | feasible |
| 3.1.1.2.2 | Demodulation of Forward Traffic Channel in Multi-path Fading Channel | feasible |
| 3.1.1.2.3 | Decision of Power Control Bit for Channels belonging to Different Power Control Sets during Soft Handoff | feasible |
| 3.1.1.2.4 | Decision of Power Control Bit for Channels belonging to the Same Power Control Set | feasible |
| 3.1.1.2.5 | Demodulation of Reverse Power Control Channel during Soft Handoff |  |
| 3.1.1.3.1 | Receiver Sensitivity and Dynamic Range | feasible |
| 3.1.1.3.2 | Single Tone Desensitization | feasible |
| 3.1.1.3.3 | Intermodulation Spurious Response Attenuation | feasible |
| 3.1.1.3.4 | Adjacent Channel Selectivity |  |
| 3.1.1.3.5 | Receiver Blocking Characteristics |  |
|  | Limitations on Emissions |  |
| 3.1.1.4.1 | Conducted Spurious Emissions | feasible |
| 3.1.1.4.2 | Radiated Spurious Emissions |  |
|  | Transmitter Minimum Standards |  |
| 3.1.2.1.1 | Frequency Coverage |  |
| 3.1.2.1.2 | Frequency Accuracy | feasible |
|  | Modulation Requirements |  |
| 3.1.2.2.1 | Time Reference | feasible |
| 3.1.2.2.2 | Waveform Quality and Frequency Accuracy | feasible |
| 3.1.2.2.3 | Redundant ACK Transmission |  |
|  | RF Output Power Requirements |  |
| 3.1.2.3.1 | Range of Open Loop Output Power | feasible |
| 3.1.2.3.2 | Time Response of Open Loop Power Control | feasible |
| 3.1.2.3.3 | Range of Closed Loop Power Control | feasible |
| 3.1.2.3.4 | Maximum RF Output Power | feasible |
| 3.1.2.3.5 | Minimum Controlled Output Power | feasible |
| 3.1.2.3.6 | Standby Output Power | feasible |
| 3.1.2.3.7 | RRI Channel Output Power | feasible |
|  | Code Domain Power |  |
| 3.1.2.3.8.1 | DRC Channel Output Power | feasible |
| 3.1.2.3.8.2 | ACK Channel Output Power | feasible |
| 3.1.2.3.8.3 | Data Channel Output Power | feasible |
|  | Limitations on Emissions |  |
| 3.1.2.4.1 | Conducted Spurious Emissions | feasible |
|  | Occupied Bandwidth |  |
| 3.1.2.4.3 | Occupied Bandwidth |  |
|  | MAC Layer Minimum Standards |  |
| 4.1.1.1 | Control Channel Supervision |  |
|  | Forward Traffic Channel MAC Protocol |  |
| 4.2.1.1 | Data Rate Control Performance |  |
| 4.2.1.2 | Demodulation of Forward Traffic Channel during Soft Handoff |  |
| 4.2.1.3 | Supervision Procedures in Variable Rate State |  |
|  | Access Channel MAC Protocol |  |
| 4.3.1.1 | Access Probes Output Power | feasible |
|  | Reverse Traffic Channel MAC Protocol |  |
| 4.4.1.1 | Reverse Traffic Channel Initial Transmit Power |  |
| 4.4.1.2 | Reverse Traffic Channel Transmit Power |  |
| 4.4.1.3 | Demodulation of the Reverse Activity Channel |  |
| 4.4.1.4 | Reverse Traffic Channel MAC rate transition probabilities |  |
|  | Key Exchange Protocol |  |
| 5.1.1.1 | Key Signature Computation time |  |
|  | Idle State |  |
| 6.1.1.1.1 | Idle Handoff in Continuous Operation Mode |  |
| 6.1.1.1.2 | Idle Handoff in Slotted Operation Mode |  |
| 6.1.1.1.3 | Idle Handoff in Suspended Operation Mode |  |
|  | Connected State |  |
| 6.1.1.2.1 | Neighbor Set Pilot Detection and Incorrect Detection in Soft Handoff |  |
| 6.1.1.2.2 | Candidate Set Pilot Detection and Incorrect Detection in Soft Handoff |  |
| 6.1.1.2.3 | Active Set Pilot Loss Detection in Soft Handoff |  |
| 6.1.1.2.4 | Hard Handoff to another Frequency |  |
|  | Environmental Requirements |  |
| 10.1 | Temperature and Power Supply Voltage | feasible |
| 10.2 | High Humidity | feasible |
| 10.3 | Vibration Stability | feasible |
| 10.4 | Shock Stability | feasible |